Like peas in a pod: the HP Series from HAMM

The next generation of pneumatic tyre rollers

With the HP series, HAMM is presenting a totally new generation of pneumatic tyre rollers at Bauma 2019. The model series is packed with technical innovations and further developments. For example, with the HP series HAMM has developed a common platform as the basis for all model variants in every market throughout the world. In addition, there are important innovations affecting the operator's platform, the water and additive sprinkling system as well as the ballasting.

In the 1960s, HAMM, the pioneer in the compaction sector, launched the revolutionary GRW and has continued to develop it up to the present day. Today, a good 50 years later, the Tirschenreuth-based compaction specialists are again launching a new generation of this roller type: the HP series.

One platform for all markets

The new pneumatic tyre roller generation will gradually replace the existing GRW series throughout the world in the course of the next two years. The first models, the HP 280i and HP 280, have already been available since the beginning of 2019. It is envisaged that all models and variants will be available worldwide by the end of the year. As before, HAMM will produce the pneumatic tyre rollers in different basic versions: on the one hand the type HP 180 and HP 180i models with operating weights of 8 t to a maximum of 18 t and on the other, the HP 280 and HP 280i models with operating weights between 10 t and a maximum of 28 t. Here, HAMM is providing the individual markets with the particular models to suit the locally demanded weight classes and equipment variants – taking the locally applicable emission regulations into account at all times.

Operator's platform with first-class field of vision

As is customary with HAMM, the new HP series pneumatic tyre rollers also offer a modern, spacious operator's platform or panoramic cabin with maximum visibility over the machine and the construction site. In this respect all models comply with the new ISO 5006-2017 operator’s field of view standard which is markedly stricter than the previous provisions.

In terms of machine design, HAMM is sticking with the proven asymmetrical frame concept in the HP series. This is not only a hallmark, it is also a quality advantage because it follows the track offset between the front and rear wheel sets and always allows a clear view of the outer flanks of the front and rear wheels.

Clear operating concept

Clarity and an unimpeded overview also prevail on the operator's platform. For example, on all models the operation is designed to be totally language-neutral and is very easy to learn. The minimal number of switches on the operator’s platform is intelligently arranged. User guidance is assisted by the unambiguous colour-coded grouping. Drive control is via a joystick ergonomically positioned on the armrest. Numerous other details in the driver's cab ensure abundant comfort and a healthy working posture for the driver.

Flexible ballasting as standard

Another feature that users have come to expect from HAMM is the flexibility in terms of ballasting: the operating weight is quickly adjustable to suit the asphalt type, layer thickness and application. Here, the HP series offers considerable latitude with its variable ballasting concept. For example, prefabricated steel, magnetite or concrete ballast bodies can be inserted into or removed from the large ballast compartments between the wheel sets using a fork-lift truck, for example. This can be carried out in the user's workshop or on the construction site. HAMM offers various ballast body kits for this purpose allowing different weights to be achieved. Because the ballast spaces are located in the centre of the machine, the weight is always evenly distributed over both axles – perfect conditions for optimum compaction quality. Additional ballasting of up to 17 t is possible, depending on the basic weight.

Highest possible operational safety included

In addition, the HP series also scores in terms of safety. Drivers can always reach the brake pedal quickly and reliably because it is an integral part of the seat-operating unit on all HP series models. This makes for the maximum possible operational safety because the brake is always easily accessible, irrespective of the seat position. This aspect is especially important particularly for heavy rollers weighing well over 20 t.

Generous tank volumes for high productivity

The new HP series pneumatic tyre rollers are visually identifiable by the large, newly designed water tank. Like the diesel tank, it is generously dimensioned so that the machines have sufficient capacity for a full working day without stopping to refill. For this reason alone, the rollers are already extremely productive.

Supplementary water tank: ballasting and water reserve

Hamm also offers a supplementary water tank with a volume of 1,500 l for all HP series models. It may be used as a water reservoir for the sprinkling system, but also serves as an easily quantifiable, flexible supplementary weight.

Operation of the supplementary tank is remarkably easy because it is connected to the basic water tank by means of inter-communicating pipes. To facilitate fast refilling, the developers have provided a C-pipe connection to complement conventional filling. It enables the entire tank to be filled in around three minutes. A pump is also fitted to allow complete discharge.

Innovative additive sprinkling system

HAMM has also optimised the additive sprinkler system for the new HP series due to the profusion of special asphalts that tend to be difficult to process.

High-quality compaction of these construction materials calls for sprinkling of the pneumatic tyres with an additive.

Previously, the additive concentrate was mixed with water and then filled into a separate tank on the roller. On the new machines, the additive concentrate is simply filled without premixing. The dosage from the large additive tank with level monitoring can be controlled from the operator's platform in various addition levels. The roller doses and mixes the additive during the compaction in accordance with the specification. This also brings about an increase in quality, because HAMM avoids separation of the additive-water mixture by virtue of the design. And last but not least, swapping from water to additive can easily be done at any time directly from the operator's platform.

Perfect maintenance – with safety assured

When it comes to maintenance, the developers have also attached importance to efficiency and safety. For example, all daily maintenance items are situated on the right-hand side of the engine. Engine access has also been intelligently and safely designed on the HP series: the steps are integrated in the outer contour and are opened outwards at a stroke. Because they visually protrude in this position, the operator cannot forget to retract the steps on conclusion of the maintenance work.

Model overview:

| Model | Projected availability | Min / max. weight | Engine output (kW) / emission level |
| --- | --- | --- | --- |
| HP 180 | From mid 2019 | 8 t / 18 t | 85 kW / Tier 3 |
| HP 180i | From end 2019 | 8 t / 18 t | 55.4 kW / Tier 4f or EU 5 |
| HP 280 | Since October 2018 | 10 t / 28 t | 85 kW / Tier 3 |
| HP 280i | Since end 2018 | 10 t / 28 t | 85 kW / Tier 4f |

Photos:

|  | HAMM_Serie HP_01 Like peas in a pod: the different models in the new HP series from HAMM (from left to right: HP 180, HP 280, HP 280i) are all based on the same platform. |
| --- | --- |
|  | HAMM_Serie HP_02 In the new HP series pneumatic rollers from HAMM, the weight can be simply and flexibly adjusted using prefabricated ballast bodies. The ballast spaces can be filled with various materials enabling the overall weight to be varied in precisely graduated stages. |
|  | HAMM_Serie HP_03 The 85 kW HP 180 (Tier 3) from HAMM will be available from the middle of 2019. |
|  | HAMM_Serie HP_04 The HP series rollers from HAMM are suitable for earthwork and asphalt paving construction. The HP 280i model (85 kW, Tier 4final, left in the picture) is already available. The power is provided by a 4-cylinder DEUTZ diesel engine. |
|  | HAMM_Serie HP_05 The operator's platform of the HAMM HP series has been optimised according to ergonomic principles: each driver can individually adjust the seat and armrests and the tilt of the dashboard with steering wheel to their personal preferences. In addition, the seat-operating unit can be moved to the outer edge of the platform and optionally rotated by 90° in either direction. |
|  | HAMM_Serie HP_06 The fold-out service steps are one of the many ingenious details contributing to the highly economical and efficient operation of the HP series. |

| Further information is available from: HAMM AG Gottfried Beer Hammstraße 1 95643 Tirschenreuth Germany Telephone: +49 (0) 9631 80 - 195 / 187 Fax: +49 (0) 9631 80 - 143 E-mail: gbeer@hamm.eu Internet: www.hamm.eu |  |
| --- | --- |