Wirtgen | Flexibility and Precision Guarantee the Right Material Size

With the Vario Impact Sizer, Wirtgen Offers a Powerful System for Material Size Management for the Surface Miner 220 SM(i) 3.8

Compliance with the required material size is a decisive factor for ensuring efficient processes in opencast mining operations. With the Vario Impact Sizer (VIS), Wirtgen offers a system that scores top marks in terms of flexibility, precision, and adaptability. It enables machine operators to reliably control the maximum particle size and particle size distribution, even under constantly changing operating conditions. Extensive evaluation of the system in practice has confirmed its effectiveness, particularly in coal and bauxite mining operations.

Precise Selectable Settings Deliver the Required Maximum Particle size

The core of the VIS is a split baffle plate mounted directly on the cutting drum housing. This plate is equipped with two primary adjustment mechanisms that enable precise control of the material that passes through the system. The variable gap between the plate and the cutting drum is the first adjustment mechanism: this distance can be adjusted in five stages to control the amount of additional crushing and thus change the particle size. A smaller gap results in extra crushing and thus finer material, while a larger gap results in coarser material. The adjustment is made by simply enlarging or reducing the gap and securing the plate with pins, which takes around ten minutes. The second adjustment mechanism relates to the holes in the scraper. Depending on the specific requirements of the operation, they can be opened or closed in four different settings by means of covers. These two mechanisms complement the basic adjustment options of the Surface Miner like the advance speed of the machine, the speed of the cutting drum, and the cutting depth. In combination with the VIS, this results in an extremely high level of control over the maximum particle size.

The Vario Impact Sizer: Durable and Reliable

Machines in cut-to-ground or windrow applications often produce material that exceeds the required maximum particle size. This is where the VIS comes in – the system is designed to fulfill precisely this requirement by influencing and limiting the maximum particle size. Finer material passes through the sizer and leaves the crushing process while coarser material is crushed until it can also pass the sizer. The VIS has proven itself in both coal and bauxite mining applications and effectively controls the size of the material produced to ensure that it meets the specified requirements. One of the advantages of the VIS is its seamless integration into the machine. The VIS is mounted directly on the cutting drum housing and integrates seamlessly into the material processing process. Furthermore, the system is highly adaptable. The variable settings enable reliable crushing of a wide range of rocks and minerals. The VIS is particularly suitable for brittle and porous rocks and minerals such as coal, bauxite, or chalk. The robust construction of the VIS ensures that it can withstand the harsh conditions in mining and meets all requirements for durability and stability, even at high throughput rates.

Use in Opencast Mining: Fine Control via Machine and VIS Settings

The VIS is available for the 220 SM(i) 3.8. Particle size control is achieved through the interaction of standard machine settings and the VIS settings. The standard machine settings include the machine’s advance speed from 0 to 84 m/min. The cutting drum speed is also adjustable with six standard settings on board. The cutting depth is adjustable from of 0 to 350 mm. The settings of the VIS consist of the five baffle plate gap settings from 22 mm to fully open and the four cover plate opening settings, which can be adjusted from fully closed to fully open. This wide range of combinations offers almost unlimited possibilities for influencing the maximum particle size and the particle size distribution.

Practical Examples from Coal and Bauxite Mining Applications

In the meantime, the performance of the VIS has been evaluated in various mining regions. In coal mining in India, the system showed improvements in the control of coal particle size. Specific settings were used to achieve a target size of less than 100 mm. In bauxite mining, the VIS delivered impressive results in the Boffa and Boké regions of Guinea. During mining in Boffa, the sizer was opened fully to enable a cutting depth of 300 mm and a high advance speed, enabling maximum performance for the achievement of high productivity. In Boké, the settings were adjusted to a material size of less than 100 mm, demonstrating the system’s ability to adapt to different types of bauxite and different crushing behaviors. Clearly noticeable in practice: the settings have to be carefully adapted to the requirements since additional crushing reduces the throughput rate.

The Vario Impact Sizer Assists Operators in Numerous Ways

The VIS provides machine operators with a tool that gives them exceptionally precise control over the maximum particle size. The system settings have a direct impact on key performance parameters: particle size distribution, production output, fuel consumption, tool wear, and stress on parts and components. By specifically adjusting the configuration, the operator can always achieve an optimal balance between material requirements, process stability, and overall cost-efficiency. The VIS thus confirms its role as an effective tool for efficient material size control in contemporary opencast mining scenarios.

Photos:

W_pic_220SM-3-8_00027_HI

The Surface Miner 220 SM(i) mining coal using the windrow method.

W_pic_VIS_all_cover_plates_0925_0001

The core component of the Vario Impact Sizer is a split baffle plate mounted directly on the cutting drum housing.

W_pic_VIS_demonstration-only-left-open-right-closed_0925_0001

For demonstration only: the plate on the left is open; the plate on the right is closed.

Note: the photographs shown here are only previews. If you wish to publish them in other media, please download the higher resolution (300 dpi) versions from the Wirtgen Group websites.

For more information, please contact us at:

WIRTGEN GROUP

Public Relations

Reinhard-Wirtgen-Straße 2

53578 Windhagen

Germany

Phone: +49-2645-131-1966

Fax: +49-2645-131-499

E-mail: PR@wirtgen-group.com

www.wirtgen-group.com