Vögele │ Efficient asphalt paving with the Edge Detection edge follower

Automated screed width control

The lead contractor used the Edge Detection laser sensor to automatically control the screed width of their Dash 5 paver along a joint edge. This ensured precise, resource-efficient paving and reduced the operators’ workload. Edge Detection is part of the Vögele AutoTrac steering and screed width control system.

Pavement rehabilitation with two mobile feeders and two road pavers from Vögele

The B535 in Baden-Württemberg is an important bypass. Due to the damaged surface, a 1.6 km section near Heidelberg-Schwetzingen was badly in need of rehabilitation. To enable precise and rapid renewal of the two-lane section, lead contractor Eiffage Infra-Bau SE decided to use staggered hot to hot paving with two mobile feeders and two pavers from Vögele in combination with the Edge Detection edge follower. On the site, the contractor first milled off the damaged asphalt pavement down to the base layer. For the renewal of the 10 cm binder course and the 4 cm surface layer, the contractor used two latest-generation Vögele pavers, a SUPER 1900-5 X and a SUPER 2100-5 X. Two MT 3000-2i Offset mobile feeders ensured an uninterrupted supply of asphalt mix to each of the two pavers.

Hot to hot paving with Edge Detection

The paving crew first paved a strip alongside the central safety barrier with the SUPER 1900-5 X. The SUPER 2100-5 X equipped with the Edge Detection edge follower followed on behind, parallel to the first paver. The laser sensor was mounted on the left side plate of the AB 600 paving screed and used the joint edge of the first strip as a reference datum for the automatic control of the screed width. In concrete terms, it works like this: the sensor scans the area in front of the side plate within a 70-degree measuring range and detects edges with a rising or falling profile of at least 2 cm. When the screed operator activates the corresponding function, the respective screed extension automatically follows the edge. ‘This allowed us to lay down the second strip very smoothly, precisely and quickly’, says paver operator Michael Wenz.

Edge Detection assists the operators

On several sections of the federal highway, the paving width varied between 8 m and 11.80 m. With a conventional paving process, the screed operator would therefore have had to manually control the extension or retraction of the screed extensions. However, thanks to Edge Detection, the screed width was automatically adjusted to pave exactly the width defined by the detected edge. This allowed the screed operator to concentrate on other aspects of the job, such as assuring the paving quality. When working on sites alongside moving traffic, automated screed width control also helps prevent safety risks by eliminating the need for operators to work in hazardous areas. Another benefit of paving with the Edge Detection sensor: the automated regulation of the screed width meant that exactly the right amount of asphalt mix was used to achieve an ideal join between the strips laid down by the two pavers. ‘The reliable edge following by the Edge Detection sensor enables us to avoid paving more than the actual width required. That saves material and cuts our costs’, emphasises Wenz

Photos:

JV_JR_Edge_Detection_001_PR
Pavement rehabilitation on a federal highway: for the hot to hot paving with two latest-generation Vögele pavers, the lead contractor made use of the new Edge Detection edge follower.

JV_JR_Edge_Detection_002_PR

The Vögele Edge Detection laser sensor scans across a measuring range of 70 degrees in front of the side plate and detects edges with a rising or falling profile of at least 2 cm.

JV_JR_Edge_Detection_003_PR

The Vögele SUPER 2100-5 X paver with the edge follower followed on behind, parallel to the SUPER 1900-5 X. The Edge Detection sensor used the joint edge of the first strip as a reference for automatic screed width control.

JV_JR_Edge_Detection_004_PR

Easy handling: after connecting up the Edge Detection sensor, all the operator has to do is activate the AutoTrac app on the touchscreen display of the paver operator’s control console.

Please note: the photographs shown here are only previews. If you wish to publish them in other media, please download the higher resolution (300 dpi) versions from the link provided here.

For further information, please contact us at:

WIRTGEN GROUP

Public Relations

Reinhard-Wirtgen-Straße 2

53578 Windhagen

Germany

Phone: 	+49 (0)2645 131 1966

Fax: 	+49 (0)2645 131 499

Email: 	PR@wirtgen-group.com

www.wirtgen-group.com